Appendix: Complete Derivations from First Principles

This appendix provides rigorous, step-by-step derivations of all fundamental relationships discussed in the main text. The goal is to build the entire framework from first principles, showing explicitly how each equation follows from the previous, and how the intrinsic-extrinsic structure emerges naturally at every scale.

# A. Foundations: Force, Source, and Field

## A.1 Newton's Second Law

We begin with the most fundamental dynamical statement in classical mechanics. Newton's Second Law states that the acceleration of a body is proportional to the net force acting upon it and inversely proportional to its mass:

F = ma    (A.1)

where:

F: the net force acting on the body [N = kg·m/s²]

m: the inertial mass of the body [kg]

a: the acceleration of the body [m/s²]

Rearranging to isolate acceleration:

a = F/m    (A.2)

This form reveals a fundamental structure: the response (acceleration) equals the drive (force) divided by the intrinsic property (mass). Equivalently:

Response = Drive / (Intrinsic Property)    (A.3)

or in product form:

Drive = (Intrinsic Property) × Response    (A.4)

## A.2 Coulomb's Law and the Electric Field

For a point charge q in an electric field E, the electrostatic force is:

F = qE    (A.5)

where:

F: the electrostatic force [N]

q: the electric charge [C = Coulombs]

E: the electric field [N/C = V/m]

Comparing (A.1) and (A.5), we identify a parallel structure:

F = ma    ↔    F = qE    (A.6)

In both cases, Force = Source × Field, where:

- In mechanics: Source = mass (m), Field = acceleration (a)

- In electrostatics: Source = charge (q), Field = electric field (E)

The units confirm this analogy:

[a] = N/kg = m/s²

[E] = N/C = V/m

Both have dimensions of [Force]/[Source], which we may call the generalized field strength.

## A.3 Newton's Law of Universal Gravitation

Consider two point masses M and m separated by distance R. Newton's gravitational law states:

F = G·Mm/R²    (A.7)

where:

G: the universal gravitational constant, G ≈ 6.674 × 10⁻¹¹ N·m²/kg²

M: the source mass creating the gravitational field [kg]

m: the test mass experiencing the field [kg]

R: the separation between mass centers [m]

To isolate the field created by M, divide both sides by m:

F/m = G·M/R²    (A.8)

By Newton's Second Law, F/m = a, so:

g = G·M/R²    (A.9)

where g is the gravitational field strength (gravitational acceleration) at distance R from mass M. This field is extrinsic to the test mass m—it is created entirely by the source mass M and exists independently of whether m is present.

The force on m is then:

F = mg    (A.10)

which has the same Source × Field structure as (A.1) and (A.5).

## A.4 Coulomb's Law for Two Charges

Similarly, for two point charges Q and q separated by distance R:

F = k·Qq/R²    (A.11)

where k ≈ 8.99 × 10⁹ N·m²/C² is Coulomb's constant. Dividing by q:

E = k·Q/R²    (A.12)

This is the electric field created by charge Q at distance R. The force on q is:

F = qE    (A.13)

## A.5 The Intrinsic-Extrinsic Decomposition

In both gravitational and electrostatic cases, we have:

Force = (Intrinsic Source) × (Extrinsic Field)    (A.14)

The source (m or q) is intrinsic—it is a property of the object experiencing the force. The field (g or E) is extrinsic—it is determined by other sources in the environment. The force itself carries information about both.

Key Observation: Given only the force F, we cannot uniquely determine the source and field separately. The same force could arise from a large source in a weak field or a small source in a strong field. This underdetermination is structural.

# B. Potential Energy and Potential

## B.1 Work and the Work-Energy Theorem

Work is defined as the energy transferred when a force moves an object through a displacement:

W = ∫ F⃗ · dr⃗    (B.1)

For a constant force F acting over displacement d at angle θ:

W = F·d·cos(θ)    (B.2)

The Work-Energy Theorem states that the net work done on an object equals its change in kinetic energy:

W_net = ΔKE = KE_f − KE_i    (B.3)

where kinetic energy is:

KE = ½mv²    (B.4)

## B.2 Conservative Forces and Potential Energy

A force is conservative if the work done depends only on the initial and final positions, not on the path taken. For such forces, we can define a potential energy function U such that:

W_conservative = −ΔU = U_i − U_f    (B.5)

The force is related to potential energy by:

F⃗ = −∇U    (B.6)

In one dimension:

F = −dU/dx    (B.7)

## B.3 Gravitational Potential Energy

For a mass m at height h above a reference level in a uniform gravitational field g:

U_grav = mgh    (B.8)

Verification: The gravitational force is F = −mg (pointing downward). Using (B.7):

F = −dU/dx = −d(mgh)/dh = −mg    ✓    (B.9)

For the general case with varying g (at distance r from mass M):

U_grav = −G·Mm/r    (B.10)

The negative sign reflects that potential energy decreases (becomes more negative) as masses approach.

## B.4 Electrostatic Potential Energy

For a charge q at position s in a uniform electric field E:

U_elec = qE·s    (B.11)

For the general case between charges Q and q at separation r:

U_elec = k·Qq/r    (B.12)

## B.5 Potential: Energy per Unit Source

We define potential as energy per unit source. This removes the intrinsic property from the energy expression, leaving only the extrinsic field information.

Gravitational Potential:

V_g = U_grav/m = gh    (uniform field)    (B.13)

V_g = −G·M/r    (point mass)    (B.14)

Units: [V_g] = J/kg = m²/s²

Electric Potential (Voltage):

V = U_elec/q = E·s    (uniform field)    (B.15)

V = k·Q/r    (point charge)    (B.16)

Units: [V] = J/C = Volts

Key insight: Potential is purely extrinsic—it characterizes the field configuration created by external sources, independent of the test source that might be placed in it.

## B.6 Energy Conservation

For a system with conservative and nonconservative forces:

KE_i + PE_i + W_nc = KE_f + PE_f    (B.17)

Rearranging:

W_nc = ΔKE + ΔPE = Δ(KE + PE)    (B.18)

If W_nc = 0 (only conservative forces):

ΔKE + ΔPE = 0    (B.19)

KE + PE = constant    (B.20)

Total mechanical energy is conserved when only conservative forces act.

# C. The Three Energy Modes and Passive Elements

## C.1 Classification of Energy Storage and Dissipation

Any dynamical system can exchange energy through three fundamental mechanisms:

- Potential Energy Storage: Energy stored in configuration (position, deformation, charge separation)

- Kinetic Energy Storage: Energy stored in motion (velocity, current, flow rate)

- Dissipation: Energy irreversibly converted to heat (friction, resistance)

These three modes correspond to three types of passive elements in physical systems.

## C.2 Mechanical Elements

Spring (Potential Storage):

A linear spring stores potential energy when deformed from equilibrium:

U_spring = ½kx²    (C.1)

where k is the spring constant [N/m] and x is the displacement from equilibrium [m]. The restoring force is:

F_spring = −kx    (C.2)

Constitutive relation (force as function of displacement):

F = kx    (magnitude)    (C.3)

Time derivative:

dF/dt = k·(dx/dt) = k·v    (C.4)

Mass (Kinetic Storage):

A mass stores kinetic energy when in motion:

KE_mass = ½mv²    (C.5)

Newton's Second Law gives the constitutive relation:

F = m·(dv/dt) = m·a    (C.6)

This says: force causes rate of change of velocity, with mass as the proportionality constant.

Damper (Dissipation):

A viscous damper dissipates energy as heat. The damping force opposes motion and is proportional to velocity:

F_damper = b·v    (C.7)

where b is the damping coefficient [N·s/m]. Power dissipated:

P_dissipated = F·v = b·v²    (C.8)

This is always positive (always dissipating), confirming energy is lost from the mechanical system.

## C.3 Electrical Elements

Capacitor (Potential Storage):

A capacitor stores energy in an electric field via charge separation:

U_capacitor = ½CV² = ½q²/C    (C.9)

where C is capacitance [F = Farads = C/V] and V is voltage [V]. The constitutive relations:

q = C·V    (C.10)

I = dq/dt = C·(dV/dt)    (C.11)

Inductor (Kinetic Storage):

An inductor stores energy in a magnetic field via current flow:

U_inductor = ½LI²    (C.12)

where L is inductance [H = Henries = V·s/A]. The constitutive relations:

λ = L·I    (flux linkage)    (C.13)

V = dλ/dt = L·(dI/dt)    (C.14)

Resistor (Dissipation):

A resistor dissipates energy as heat. Ohm's Law:

V = R·I    (C.15)

where R is resistance [Ω = Ohms = V/A]. Power dissipated:

P_dissipated = V·I = I²R = V²/R    (C.16)

## C.4 The Mechanical-Electrical Analogy

Comparing the constitutive relations reveals a precise correspondence:

F = m·(dv/dt)    ↔    V = L·(dI/dt)    (C.17)

dF/dt = k·v    ↔    I = C·(dV/dt)    (C.18)

F = b·v    ↔    V = R·I    (C.19)

The mapping is:

- Force F ↔ Voltage V (effort variables)

- Velocity v ↔ Current I (flow variables)

- Mass m ↔ Inductance L (kinetic storage)

- Compliance 1/k ↔ Capacitance C (potential storage)

- Damping b ↔ Resistance R (dissipation)

## C.5 Hydraulic Elements

Accumulator (Potential Storage):

A hydraulic accumulator stores energy via fluid compression:

U_accumulator = ½C_h·(Δp)²    (C.20)

where C_h is hydraulic compliance [m³/Pa]. Constitutive relation:

Q = C_h·(d(Δp)/dt)    (C.21)

Inertance (Kinetic Storage):

Fluid inertance stores energy in moving fluid:

U_inertance = ½I_h·Q²    (C.22)

where I_h is inertance [Pa·s²/m³]. Constitutive relation:

Δp = I_h·(dQ/dt)    (C.23)

Restriction (Dissipation):

A hydraulic restriction dissipates energy via viscous losses:

Δp = R_h·Q    (C.24)

where R_h is hydraulic resistance [Pa·s/m³].

# D. Effort, Flow, and Power

## D.1 Generalized Variables

Across all energy domains, we identify two conjugate variables:

Effort (e): The driving difference that "pushes" — voltage, force, pressure difference, gravitational potential

Flow (f): The through quantity that "moves" — current, velocity, volume flow rate, mass flow rate

These are conjugate in the sense that their product is power:

P = e · f    (D.1)

Verification across domains:

- Electrical: P = V·I [V·A = W]

- Mechanical: P = F·v [N·m/s = W]

- Hydraulic: P = Δp·Q [Pa·m³/s = W]

- Gravitational: P = V_g·ṁ [J/kg · kg/s = W]

## D.2 State Variables

Each storage element has an associated state variable that accumulates over time:

C-type (potential storage) state: The integral of flow

q_C = ∫f dt    (D.2)

- Electrical: charge q = ∫I dt

- Mechanical: displacement x = ∫v dt

- Hydraulic: volume V = ∫Q dt

I-type (kinetic storage) state: The integral of effort

p_I = ∫e dt    (D.3)

- Electrical: flux linkage λ = ∫V dt

- Mechanical: momentum p = ∫F dt

- Hydraulic: pressure impulse Π = ∫Δp dt

## D.3 Impedance

Impedance is the ratio of effort to flow at a port:

Z = e/f    (D.4)

For a purely resistive element, Z equals the resistance constant. When storage elements are present, impedance becomes frequency-dependent (complex in the Laplace or Fourier domain), because energy is temporarily stored before being returned or dissipated.

## D.4 Power Balance and Energy Accounting

At any instant, power flows into or out of each element:

P_in = Σ(e_in · f_in)    (D.5)

For a closed system:

P_stored + P_dissipated = P_input    (D.6)

where:

P_stored = d/dt(U_potential + U_kinetic)    (D.7)

P_dissipated = Σ R_i · f_i²    (D.8)

# E. The Single Forced Oscillator

## E.1 Equation of Motion

Consider a mass m connected to a fixed wall by a spring k and damper b, subject to external force F(t). Newton's Second Law:

m·ẍ + b·ẋ + k·x = F(t)    (E.1)

where:

x(t): position (unknown function of time) [m]

ẋ = dx/dt: velocity [m/s]

ẍ = d²x/dt²: acceleration [m/s²]

m: mass [kg] — intrinsic

b: damping coefficient [N·s/m] — intrinsic

k: spring constant [N/m] — intrinsic

F(t): external forcing function [N] — extrinsic

## E.2 State-Space Formulation

To convert to first-order form, define state variables:

z₁ = x    (position)    (E.2)

z₂ = ẋ    (velocity)    (E.3)

Then:

ż₁ = z₂    (E.4)

ż₂ = (1/m)[F(t) − b·z₂ − k·z₁]    (E.5)

In matrix form with state vector z = [z₁, z₂]ᵀ:

dz/dt = A_θ·z + s(t)    (E.6)

where the intrinsic matrix is:

A_θ = [0, 1; −k/m, −b/m]    (E.7)

and the extrinsic forcing vector is:

s(t) = [0; F(t)/m]ᵀ    (E.8)

## E.3 Eigenvalue Analysis

The eigenvalues of A_θ determine system behavior. The characteristic equation:

det(A_θ − λI) = 0    (E.9)

λ² + (b/m)λ + (k/m) = 0    (E.10)

Defining natural frequency ω₀ = √(k/m) and damping ratio ζ = b/(2√(km)):

λ = −ζω₀ ± ω₀√(ζ² − 1)    (E.11)

Three regimes:

- ζ < 1 (underdamped): Complex eigenvalues → oscillatory decay

- ζ = 1 (critically damped): Repeated real eigenvalues → fastest non-oscillatory decay

- ζ > 1 (overdamped): Distinct real eigenvalues → slow exponential decay

## E.4 Energy Analysis

Total mechanical energy:

E = ½mẋ² + ½kx² = ½m·z₂² + ½k·z₁²    (E.12)

Time derivative:

dE/dt = m·ẋ·ẍ + k·x·ẋ    (E.13)

Substituting from (E.1):

dE/dt = ẋ·(m·ẍ + k·x) = ẋ·(F(t) − b·ẋ) = F(t)·ẋ − b·ẋ²    (E.14)

Interpretation:

- F(t)·ẋ = power input from external forcing

- b·ẋ² = power dissipated by damper (always positive)

# F. Chain of Coupled Oscillators

## F.1 Setup

Consider n masses m₁, m₂, ..., mₙ arranged in a line. Mass i is connected to mass i+1 by a spring with stiffness k_i and damper with coefficient c_i. The first and last masses may be connected to fixed walls.

Let x_i(t) denote the displacement of mass i from equilibrium. External forces F_i(t) may act on each mass.

## F.2 Equation of Motion for Interior Mass

For an interior mass i (not at the ends), the forces are:

- Spring force from left neighbor: k_{i-1}(x_{i-1} − x_i)

- Spring force from right neighbor: k_i(x_{i+1} − x_i)

- Damping force from left: c_{i-1}(ẋ_{i-1} − ẋ_i)

- Damping force from right: c_i(ẋ_{i+1} − ẋ_i)

- External force: F_i(t)

Newton's Second Law:

m_i·ẍ_i = k_{i-1}(x_{i-1} − x_i) + k_i(x_{i+1} − x_i) + c_{i-1}(ẋ_{i-1} − ẋ_i) + c_i(ẋ_{i+1} − ẋ_i) + F_i(t)    (F.1)

## F.3 Identical Parameters

Assume identical masses m, springs k, and dampers c. For interior mass i:

m·ẍ_i = k(x_{i-1} − 2x_i + x_{i+1}) + c(ẋ_{i-1} − 2ẋ_i + ẋ_{i+1}) + F_i(t)    (F.2)

The coefficient pattern (1, −2, 1) is the discrete second derivative stencil.

## F.4 Matrix Formulation

Define the state vector:

z = [x₁, ẋ₁, x₂, ẋ₂, ..., xₙ, ẋₙ]ᵀ ∈ ℝ²ⁿ    (F.3)

The system can be written as:

dz/dt = A_θ·z + s(t)    (F.4)

where:

A_θ = [0, I; −M⁻¹K, −M⁻¹C]    (F.5)

## F.5 Mass Matrix

The mass matrix is diagonal:

M = diag(m₁, m₂, ..., mₙ)    (F.6)

For identical masses m: M = m·I_n.

## F.6 Stiffness Matrix

For a chain with nearest-neighbor coupling and fixed walls at both ends:

K_{ii} = k_{i-1} + k_i    (F.7)

K_{i,i+1} = K_{i+1,i} = −k_i    (F.8)

All other entries are zero. For identical springs k:

K = k × [2, −1, 0, ..., 0; −1, 2, −1, ..., 0; 0, −1, 2, ..., 0; ...; 0, ..., −1, 2]    (F.9)

This is the n×n tridiagonal matrix with 2 on the diagonal and −1 on the super- and sub-diagonals.

## F.7 Damping Matrix

The damping matrix C has the same tridiagonal structure as K, with c replacing k:

C_{ii} = c_{i-1} + c_i    (F.10)

C_{i,i+1} = C_{i+1,i} = −c_i    (F.11)

## F.8 Forcing Vector

The external forces enter through:

s(t) = [0, F₁(t)/m₁, 0, F₂(t)/m₂, ..., 0, Fₙ(t)/mₙ]ᵀ    (F.12)

The zeros correspond to the kinematic relations ẋ_i = v_i.

# G. The Continuum Limit

## G.1 Setup

Consider the chain of F.2 with identical parameters. We now derive the continuum limit by letting the number of masses n → ∞ while holding the total length L fixed.

Define:

Δx = L/n: spacing between masses

x_i = i·Δx: position of mass i along the chain

u(x,t): continuous displacement field such that u(x_i, t) ≈ x_i(t)

## G.2 Relating Lumped to Continuum Parameters

The lumped parameters must be related to material properties:

m = ρ·A·Δx    (G.1)

where ρ is mass density [kg/m³] and A is cross-sectional area [m²].

k = E·A/Δx    (G.2)

where E is Young's modulus [Pa = N/m²].

c = η·A/Δx    (G.3)

where η is a viscosity-like damping coefficient [Pa·s].

For distributed forcing:

F_i(t) = f(x_i, t)·Δx    (G.4)

where f(x,t) is force per unit length [N/m].

## G.3 Substitution

Substitute (G.1)-(G.4) into the discrete equation (F.2):

(ρAΔx)·ü_i = (EA/Δx)(u_{i-1} − 2u_i + u_{i+1}) + (ηA/Δx)(u̇_{i-1} − 2u̇_i + u̇_{i+1}) + f_i·Δx    (G.5)

Divide by Δx:

ρA·ü_i = (EA/Δx²)(u_{i-1} − 2u_i + u_{i+1}) + (ηA/Δx²)(u̇_{i-1} − 2u̇_i + u̇_{i+1}) + f_i    (G.6)

## G.4 The Discrete Second Derivative

The central difference approximation to the second derivative is:

∂²u/∂x² ≈ (u_{i-1} − 2u_i + u_{i+1})/Δx²    (G.7)

As Δx → 0, this converges to the exact second partial derivative.

## G.5 The Continuum Equation

Taking the limit Δx → 0:

ρA·∂²u/∂t² = EA·∂²u/∂x² + ηA·∂³u/∂x²∂t + f(x,t)    (G.8)

Dividing by ρA:

∂²u/∂t² = (E/ρ)·∂²u/∂x² + (η/ρ)·∂³u/∂x²∂t + f(x,t)/(ρA)    (G.9)

Define:

c² = E/ρ: wave speed squared [m²/s²]

γ = η/ρ: kinematic damping coefficient [m²/s]

g(x,t) = f(x,t)/(ρA): body force per unit mass [m/s²]

Then:

∂²u/∂t² = c²·∂²u/∂x² + γ·∂³u/∂x²∂t + g(x,t)    (G.10)

This is the damped wave equation of Kelvin-Voigt type.

## G.6 Special Cases

Undamped wave equation (γ = 0):

∂²u/∂t² = c²·∂²u/∂x² + g(x,t)    (G.11)

Rayleigh damping (if dampers connect to ground rather than between neighbors):

∂²u/∂t² + 2γ·∂u/∂t = c²·∂²u/∂x² + g(x,t)    (G.12)

## G.7 Boundary Conditions

Fixed ends (walls):

u(0,t) = u(L,t) = 0    (G.13)

Free ends:

∂u/∂x|_{x=0} = ∂u/∂x|_{x=L} = 0    (G.14)

# H. Field Evolution as Operator Equation

## H.1 First-Order Form

To cast the wave equation in the same intrinsic-extrinsic form as the finite-dimensional case, introduce:

w₁(x,t) = u(x,t)    (displacement field)    (H.1)

w₂(x,t) = ∂u/∂t    (velocity field)    (H.2)

The wave equation (G.11) becomes the system:

∂w₁/∂t = w₂    (H.3)

∂w₂/∂t = c²·∂²w₁/∂x² + g(x,t)    (H.4)

## H.2 Operator Notation

Define the state field w = [w₁, w₂]ᵀ and write:

∂w/∂t = ℒ_θ[w] + S(x,t)    (H.5)

where the intrinsic operator is:

ℒ_θ[w] = [w₂; c²·∂²w₁/∂x²]    (H.6)

and the extrinsic source is:

S(x,t) = [0; g(x,t)]    (H.7)

## H.3 Correspondence with Finite Dimensions

Compare with the finite-dimensional case dz/dt = A_θ·z + s(t):

- The state vector z becomes the state field w

- The matrix A_θ becomes the differential operator ℒ_θ

- The forcing vector s(t) becomes the source field S(x,t)

- Matrix multiplication becomes operator application (including differentiation)

## H.4 Intrinsic vs Extrinsic in the Field Equation

The intrinsic operator ℒ_θ encodes:

- Material parameters: wave speed c (from E, ρ)

- Damping: coefficient γ (from η, ρ)

- Geometry: domain [0,L], boundary conditions

The extrinsic part S(x,t) encodes:

- Distributed forcing g(x,t)

- Boundary drives (if non-homogeneous BCs)

- Initial conditions w(x,0)

# I. Conservation Laws and Constitutive Relations

## I.1 The Generic Pattern

Many field equations arise from combining:

- Conservation law: ∂u/∂t + ∇·J = S

- Constitutive relation: J = Response × Drive

The constitutive relation specifies how flux J depends on the field u and its gradients.

## I.2 Heat Conduction

Conservation of energy:

ρc_p·∂T/∂t + ∇·q = Q    (I.1)

where T is temperature, q is heat flux, ρ is density, c_p is specific heat, and Q is volumetric heat source.

Fourier's Law (constitutive):

q = −k∇T    (I.2)

where k is thermal conductivity [W/(m·K)]. The negative sign indicates heat flows from hot to cold.

Combining:

ρc_p·∂T/∂t = ∇·(k∇T) + Q    (I.3)

For constant k:

∂T/∂t = α∇²T + Q/(ρc_p)    (I.4)

where α = k/(ρc_p) is thermal diffusivity [m²/s].

Intrinsic: α (thermal diffusivity)

Extrinsic: Q (heat source), boundary temperatures

## I.3 Mass Diffusion

Conservation of mass:

∂c/∂t + ∇·J = S    (I.5)

where c is concentration [mol/m³ or kg/m³] and J is mass flux.

Fick's Law (constitutive):

J = −D∇c    (I.6)

where D is diffusivity [m²/s].

Combining:

∂c/∂t = D∇²c + S    (I.7)

## I.4 Charge Transport

Conservation of charge:

∂ρ_e/∂t + ∇·J = 0    (I.8)

where ρ_e is charge density [C/m³] and J is current density [A/m²].

Ohm's Law (constitutive):

J = σE    (I.9)

where σ is electrical conductivity [S/m = A/(V·m)] and E is electric field [V/m].

In electrostatics, E = −∇V, so:

J = −σ∇V    (I.10)

Combining:

∂ρ_e/∂t = ∇·(σ∇V)    (I.11)

## I.5 Linear Elasticity

Conservation of momentum:

ρ·∂²u/∂t² = ∇·σ + f    (I.12)

where u is displacement vector, σ is stress tensor, and f is body force density.

Hooke's Law (constitutive):

σ = C : ε    (I.13)

where C is the fourth-order stiffness tensor and ε is the strain tensor:

ε = ½(∇u + (∇u)ᵀ)    (I.14)

Combining: yields the Navier-Cauchy equations for elastic wave propagation.

## I.6 Summary Table

Each field equation has the form ∂u/∂t = ℒ_θ[u] + S:

- Heat: ∂T/∂t = α∇²T + S_T

- Diffusion: ∂c/∂t = D∇²c + S_c

- Charge: ∂ρ/∂t = ∇·(σ∇V)

- Waves: ∂²u/∂t² = c²∇²u + g

In all cases, ℒ_θ is determined by intrinsic material/geometric properties, and S encodes extrinsic forcing.

# J. Linear Response Theory

## J.1 Generalized Forces and Fluxes

In irreversible thermodynamics, transport phenomena are described by:

J_i = Σ_j L_{ij} X_j    (J.1)

where:

J_i: generalized flux (current, heat flux, mass flux, etc.)

X_j: generalized thermodynamic force (∇T, ∇μ, E, etc.)

L_{ij}: phenomenological (Onsager) transport coefficients

## J.2 Examples of Transport Coefficients

Single-flux systems:

- Thermal conduction: J_q = L_qq(−∇T/T²) → Fourier's law with k = L_qq/T²

- Electrical conduction: J_e = L_ee(−∇V/T) → Ohm's law with σ = L_ee/T

- Mass diffusion: J_m = L_mm(−∇μ/T) → Fick's law

Coupled systems:

- Thermoelectric: heat and charge flow couple through off-diagonal L_{qe}, L_{eq}

- Seebeck/Peltier effects arise from these cross-coefficients

## J.3 Onsager Reciprocal Relations

For systems near equilibrium, Onsager showed:

L_{ij} = L_{ji}    (J.2)

This symmetry reflects time-reversal invariance of microscopic dynamics.

## J.4 The Product Structure

The linear response form J = L·X is precisely:

Flux = Response × Drive    (J.3)

where L encodes intrinsic material properties and X encodes extrinsic driving forces.

# K. Causal Attribution and Underdetermination

## K.1 The Fundamental Problem

Given the product structure:

B = R × D    (K.1)

where B is observed behavior, R is intrinsic response, and D is extrinsic drive, a fundamental ambiguity arises.

For any nonzero scalar α:

B = R × D = (αR) × (D/α)    (K.2)

The same behavior B is consistent with infinitely many (R, D) pairs.

## K.2 Physical Interpretation

Consider J = σE (Ohm's law). Observing current density J, we cannot distinguish:

- High conductivity σ with weak field E

- Low conductivity σ with strong field E

- Any intermediate combination

Without independent measurement of either σ or E, the attribution is underdetermined.

## K.3 The Matrix Case

For vector systems J = L·X:

J = L·X = (L·M)·(M⁻¹·X)    (K.3)

for any invertible matrix M. The pair (L·M, M⁻¹·X) produces the same flux J.

## K.4 Resolving Ambiguity

To separate intrinsic from extrinsic contributions, one must:

- Control the drive: Apply known, calibrated forces/fields

- Hold intrinsic properties fixed: Same material, temperature, geometry

- Independent measurement: Measure σ separately via known E, then predict J for new E

This is the logic of experimental calibration and material characterization.

## K.5 The Nature-Nurture Analog

The same structure appears in quantitative genetics:

Phenotype = Genotype × Environment    (K.4)

Observed phenotypes cannot uniquely reveal genetic versus environmental contributions without controlled comparisons (twin studies, common-garden experiments).

## K.6 Epistemological Implications

The product structure implies:

- Behavior alone cannot support sharp causal attribution

- Auxiliary controls and measurements are always required

- Claims of "pure" intrinsic or extrinsic causation are structurally unjustified

This is not a limitation of measurement precision—it is a mathematical feature of product-structured models.

# L. Symbol Reference

Mechanical:

x, u: displacement [m]

v = ẋ: velocity [m/s]

a = ẍ: acceleration [m/s²]

m: mass [kg]

k: spring constant [N/m]

b, c: damping coefficient [N·s/m]

F: force [N]

ρ: mass density [kg/m³]

E: Young's modulus [Pa]

Electrical:

V: voltage [V]

I: current [A]

q: charge [C]

R: resistance [Ω]

C: capacitance [F]

L: inductance [H]

σ: conductivity [S/m]

E (field): electric field [V/m]

Thermal:

T: temperature [K]

q (flux): heat flux [W/m²]

k: thermal conductivity [W/(m·K)]

α: thermal diffusivity [m²/s]

c_p: specific heat [J/(kg·K)]

General:

e: effort (generalized driving variable)

f: flow (generalized through variable)

P = e·f: power [W]

Z = e/f: impedance

ℒ_θ: intrinsic differential operator

S: extrinsic source term

— END OF APPENDIX —