Appendix: Complete Derivations from First Principles

This appendix provides rigorous, step-by-step derivations of all fundamental relationships discussed in the main text. The goal is to build the entire framework from first principles, showing explicitly how each equation follows from the previous, and how the intrinsic-extrinsic structure emerges naturally at every scale.

# A. Foundations: Force, Source, and Field

## A.1 Newton's Second Law

We begin with the most fundamental dynamical statement in classical mechanics. Newton's Second Law states that the acceleration of a body is proportional to the net force acting upon it and inversely proportional to its mass:

F = ma    (A.1)

where:

F: the net force acting on the body [N = kg·m/s²]

m: the inertial mass of the body [kg]

a: the acceleration of the body [m/s²]

Rearranging to isolate acceleration:

a = Fm    (A.2)

This form reveals a fundamental structure: the response (acceleration) equals the drive (force) divided by the intrinsic property (mass). Equivalently:

Response = DriveIntrinsic Property    (A.3)

or in product form:

Drive = Intrinsic Property × Response    (A.4)

## A.2 Coulomb's Law and the Electric Field

For a point charge q in an electric field E, the electrostatic force is:

F = qE    (A.5)

where:

F: the electrostatic force [N]

q: the electric charge [C = Coulombs]

E: the electric field [N/C = V/m]

Comparing (A.1) and (A.5), we identify a parallel structure:

F = ma  ↔  F = qE    (A.6)

In both cases, Force = Source × Field, where:

- In mechanics: Source = mass (m), Field = acceleration (a)

- In electrostatics: Source = charge (q), Field = electric field (E)

## A.3 Newton's Law of Universal Gravitation

Consider two point masses M and m separated by distance R. Newton's gravitational law states:

F = GMmR2    (A.7)

where:

G: the universal gravitational constant, G ≈ 6.674 × 10⁻¹¹ N·m²/kg²

M: the source mass creating the gravitational field [kg]

m: the test mass experiencing the field [kg]

R: the separation between mass centers [m]

To isolate the field created by M, divide both sides by m:

Fm = GMR2    (A.8)

By Newton's Second Law, F/m = a, so:

g = GMR2    (A.9)

where g is the gravitational field strength (gravitational acceleration) at distance R from mass M. This field is extrinsic to the test mass m—it is created entirely by the source mass M and exists independently of whether m is present.

The force on m is then:

F = mg    (A.10)

which has the same Source × Field structure as (A.1) and (A.5).

## A.4 Coulomb's Law for Two Charges

Similarly, for two point charges Q and q separated by distance R:

F = kQqR2    (A.11)

where k ≈ 8.99 × 10⁹ N·m²/C² is Coulomb's constant. Dividing by q:

E = kQR2    (A.12)

This is the electric field created by charge Q at distance R. The force on q is:

F = qE    (A.13)

## A.5 The Intrinsic-Extrinsic Decomposition

In both gravitational and electrostatic cases, we have:

Force = (Intrinsic Source) × (Extrinsic Field)    (A.14)

The source (m or q) is intrinsic—it is a property of the object experiencing the force. The field (g or E) is extrinsic—it is determined by other sources in the environment. The force itself carries information about both.

Key Observation: Given only the force F, we cannot uniquely determine the source and field separately. The same force could arise from a large source in a weak field or a small source in a strong field. This underdetermination is structural.

# B. Potential Energy and Potential

## B.1 Work and the Work-Energy Theorem

Work is defined as the energy transferred when a force moves an object through a displacement:

W = ∫ F⃗ · dr⃗    (B.1)

For a constant force F acting over displacement d at angle θ:

W = Fd cos θ    (B.2)

The Work-Energy Theorem states that the net work done on an object equals its change in kinetic energy:

Wnet = ΔKE = KEf − KEi    (B.3)

where kinetic energy is:

KE = 12mv2    (B.4)

## B.2 Conservative Forces and Potential Energy

A force is conservative if the work done depends only on the initial and final positions, not on the path taken. For such forces, we can define a potential energy function U such that:

Wconservative = −ΔU = Ui − Uf    (B.5)

The force is related to potential energy by:

F⃗ = −∇U    (B.6)

In one dimension:

F = −dUdx    (B.7)

## B.3 Gravitational Potential Energy

For a mass m at height h above a reference level in a uniform gravitational field g:

Ugrav = mgh    (B.8)

Verification: The gravitational force is F = −mg (pointing downward). Using (B.7):

F = −dUdh = −d(mgh)dh = −mg  ✓    (B.9)

For the general case with varying g (at distance r from mass M):

Ugrav = −GMmr    (B.10)

The negative sign reflects that potential energy decreases (becomes more negative) as masses approach.

## B.4 Electrostatic Potential Energy

For a charge q at position s in a uniform electric field E:

Uelec = qEs    (B.11)

For the general case between charges Q and q at separation r:

Uelec = kQqr    (B.12)

## B.5 Potential: Energy per Unit Source

We define potential as energy per unit source. This removes the intrinsic property from the energy expression, leaving only the extrinsic field information.

Gravitational Potential:

Vg = Ugravm = gh    (B.13)

Vg = −GMr   (point mass)    (B.14)

Units: [Vg] = J/kg = m²/s²

Electric Potential (Voltage):

V = Uelecq = Es    (B.15)

V = kQr   (point charge)    (B.16)

Units: [V] = J/C = Volts

Key insight: Potential is purely extrinsic—it characterizes the field configuration created by external sources, independent of the test source that might be placed in it.

## B.6 Energy Conservation

For a system with conservative and nonconservative forces:

KEi + PEi + Wnc = KEf + PEf    (B.17)

Rearranging:

Wnc = ΔKE + ΔPE    (B.18)

If W_nc = 0 (only conservative forces):

ΔKE + ΔPE = 0    (B.19)

KE + PE = constant    (B.20)

Total mechanical energy is conserved when only conservative forces act.

# C. The Three Energy Modes and Passive Elements

## C.1 Classification of Energy Storage and Dissipation

Any dynamical system can exchange energy through three fundamental mechanisms:

- Potential Energy Storage: Energy stored in configuration (position, deformation, charge separation)

- Kinetic Energy Storage: Energy stored in motion (velocity, current, flow rate)

- Dissipation: Energy irreversibly converted to heat (friction, resistance)

## C.2 Mechanical Elements

Spring (Potential Storage):

A linear spring stores potential energy when deformed from equilibrium:

Uspring = 12kx2    (C.1)

where k is the spring constant [N/m] and x is the displacement from equilibrium [m]. The restoring force is:

Fspring = −kx    (C.2)

Constitutive relation (magnitude):

F = kx    (C.3)

Time derivative form:

dFdt = kdxdt = kv    (C.4)

Mass (Kinetic Storage):

A mass stores kinetic energy when in motion:

KEmass = 12mv2    (C.5)

Newton's Second Law gives the constitutive relation:

F = mdvdt = ma    (C.6)

Damper (Dissipation):

A viscous damper dissipates energy as heat. The damping force opposes motion:

Fdamper = bv    (C.7)

where b is the damping coefficient [N·s/m]. Power dissipated:

Pdissipated = Fv = bv2    (C.8)

## C.3 Electrical Elements

Capacitor (Potential Storage):

Ucapacitor = 12CV2 = 12q2C    (C.9)

Constitutive relations:

q = CV    (C.10)

I = dqdt = CdVdt    (C.11)

Inductor (Kinetic Storage):

Uinductor = 12LI2    (C.12)

Constitutive relations:

λ = LI   (flux linkage)    (C.13)

V = dλdt = LdIdt    (C.14)

Resistor (Dissipation):

V = RI    (C.15)

Pdissipated = VI = I2R = V2R    (C.16)

## C.4 The Mechanical-Electrical Analogy

Comparing the constitutive relations reveals a precise correspondence:

F = mdvdt  ↔  V = LdIdt    (C.17)

dFdt = kv  ↔  I = CdVdt    (C.18)

F = bv  ↔  V = RI    (C.19)

The mapping is:

- Force F ↔ Voltage V (effort variables)

- Velocity v ↔ Current I (flow variables)

- Mass m ↔ Inductance L (kinetic storage)

- Compliance 1/k ↔ Capacitance C (potential storage)

- Damping b ↔ Resistance R (dissipation)

# D. Effort, Flow, and Power

## D.1 Generalized Variables

Across all energy domains, we identify two conjugate variables:

Effort (e): The driving difference that "pushes" — voltage, force, pressure difference

Flow (f): The through quantity that "moves" — current, velocity, volume flow rate

These are conjugate in the sense that their product is power:

P = e · f    (D.1)

Verification across domains:

- Electrical: P = V·I [V·A = W]

- Mechanical: P = F·v [N·m/s = W]

- Hydraulic: P = Δp·Q [Pa·m³/s = W]

## D.2 State Variables

Each storage element has an associated state variable that accumulates over time:

C-type (potential storage) state: The integral of flow

qC = ∫ f dt    (D.2)

- Electrical: charge q = ∫I dt

- Mechanical: displacement x = ∫v dt

- Hydraulic: volume V = ∫Q dt

I-type (kinetic storage) state: The integral of effort

pI = ∫ e dt    (D.3)

- Electrical: flux linkage λ = ∫V dt

- Mechanical: momentum p = ∫F dt

- Hydraulic: pressure impulse Π = ∫Δp dt

## D.3 Impedance

Impedance is the ratio of effort to flow at a port:

Z = ef    (D.4)

# E. The Single Forced Oscillator

## E.1 Equation of Motion

Consider a mass m connected to a fixed wall by a spring k and damper b, subject to external force F(t). Newton's Second Law:

md2xdt2 + bdxdt + kx = F(t)    (E.1)

or more compactly:

mẍ + bẋ + kx = F(t)

where:

x(t): position (unknown function of time) [m]

ẋ = dx/dt: velocity [m/s]

ẍ = d²x/dt²: acceleration [m/s²]

m, b, k: intrinsic parameters (mass, damping, stiffness)

F(t): extrinsic forcing function [N]

## E.2 State-Space Formulation

To convert to first-order form, define state variables:

z1 = x   (position)    (E.2)

z2 = ẋ   (velocity)    (E.3)

Then:

ż1 = z2    (E.4)

ż2 = 1m[F(t) − bz2 − kz1]    (E.5)

In matrix form with state vector z = [z₁, z₂]ᵀ:

dzdt = Aθz + s(t)    (E.6)

where the intrinsic matrix is:

Aθ = [0, 1; −k/m, −b/m]    (E.7)

and the extrinsic forcing vector is:

s(t) = [0, F(t)/m]ᵀ    (E.8)

## E.3 Eigenvalue Analysis

The eigenvalues of A_θ determine system behavior. The characteristic equation:

det(Aθ − λI) = 0    (E.9)

λ2 + bmλ + km = 0    (E.10)

Defining natural frequency ω₀ = √(k/m) and damping ratio ζ = b/(2√(km)):

λ = −ζω0 ± ω0ζ2 − 1    (E.11)

Three regimes:

- ζ < 1 (underdamped): Complex eigenvalues → oscillatory decay

- ζ = 1 (critically damped): Repeated real eigenvalues → fastest non-oscillatory decay

- ζ > 1 (overdamped): Distinct real eigenvalues → slow exponential decay

## E.4 Energy Analysis

Total mechanical energy:

E = 12mẋ2 + 12kx2    (E.12)

Time derivative:

dEdt = mẋẍ + kxẋ    (E.13)

Substituting from (E.1):

dEdt = ẋ(mẍ + kx) = ẋ(F(t) − bẋ) = F(t)ẋ − bẋ2    (E.14)

Interpretation:

- F(t)·ẋ = power input from external forcing

- bẋ² = power dissipated by damper (always positive)

# F. Chain of Coupled Oscillators

## F.1 Setup

Consider n masses arranged in a line. Mass i is connected to mass i+1 by a spring with stiffness kᵢ and damper with coefficient cᵢ. Let xᵢ(t) denote the displacement of mass i from equilibrium.

## F.2 Equation of Motion for Interior Mass

For an interior mass i (not at the ends), Newton's Second Law:

miẍi = ki−1(xi−1 − xi) + ki(xi+1 − xi) + Fi(t)    (F.1)

## F.3 Identical Parameters

Assume identical masses m, springs k, and dampers c. For interior mass i:

mẍi = k(xi−1 − 2xi + xi+1) + c(ẋi−1 − 2ẋi + ẋi+1) + Fi(t)    (F.2)

The coefficient pattern (1, −2, 1) is the discrete second derivative stencil.

## F.4 Matrix Formulation

Define the state vector:

z = [x1, ẋ1, x2, ẋ2, ..., xn, ẋn]ᵀ ∈ ℝ²ⁿ    (F.3)

The system can be written as:

dzdt = Aθz + s(t)    (F.4)

where:

Aθ = [0, I; −M−1K, −M−1C]    (F.5)

## F.5 Stiffness Matrix

For a chain with nearest-neighbor coupling and fixed walls:

Kii = ki−1 + ki    (F.6)

Ki,i+1 = Ki+1,i = −ki    (F.7)

For identical springs k, this gives the tridiagonal matrix with 2k on diagonal and −k on off-diagonals.

# G. The Continuum Limit

## G.1 Setup

We derive the continuum limit by letting n → ∞ while holding total length L fixed. Define:

Δx = L/n: spacing between masses

u(x,t): continuous displacement field such that u(xᵢ, t) ≈ xᵢ(t)

## G.2 Relating Lumped to Continuum Parameters

m = ρAΔx    (G.1)

where ρ is mass density [kg/m³] and A is cross-sectional area [m²].

k = EAΔx    (G.2)

where E is Young's modulus [Pa].

c = ηAΔx    (G.3)

where η is a viscosity-like damping coefficient [Pa·s].

## G.3 The Discrete Second Derivative

The central difference approximation:

∂2u∂x2 ≈ ui−1 − 2ui + ui+1Δx2    (G.4)

## G.4 The Continuum Equation

Substituting and taking Δx → 0:

ρA∂2u∂t2 = EA∂2u∂x2 + ηA∂3u∂x2∂t + f(x,t)    (G.5)

Dividing by ρA:

∂2u∂t2 = c2∂2u∂x2 + γ∂3u∂x2∂t + g(x,t)    (G.6)

where:

c² = E/ρ: wave speed squared [m²/s²]

γ = η/ρ: kinematic damping [m²/s]

g = f/(ρA): body force per unit mass [m/s²]

This is the damped wave equation of Kelvin-Voigt type.

## G.5 Undamped Wave Equation

Setting γ = 0:

∂2u∂t2 = c2∂2u∂x2 + g(x,t)    (G.7)

# H. Field Evolution as Operator Equation

## H.1 First-Order Form

Introduce the state field w = [u, ∂u/∂t]ᵀ = [w₁, w₂]ᵀ. The wave equation becomes:

∂w1∂t = w2    (H.1)

∂w2∂t = c2∂2w1∂x2 + g(x,t)    (H.2)

## H.2 Operator Notation

∂w∂t = ℒθ[w] + S(x,t)    (H.3)

where the intrinsic operator is:

ℒθ[w] = [w2; c2∂2w1∂x2]    (H.4)

and the extrinsic source is:

S(x,t) = [0; g(x,t)]    (H.5)

This is the infinite-dimensional analogue of dz/dt = A_θ z + s(t).

# I. Conservation Laws and Constitutive Relations

## I.1 The Generic Pattern

Many field equations arise from combining:

- Conservation law: ∂u/∂t + ∇·J = S

- Constitutive relation: J = Response × Drive

## I.2 Heat Conduction

Conservation of energy:

ρcp∂T∂t + ∇·q = Q    (I.1)

Fourier's Law:

q = −k∇T    (I.2)

Heat equation:

∂T∂t = α∇2T + Qρcp    (I.3)

where α = k/(ρc_p) is thermal diffusivity [m²/s].

## I.3 Mass Diffusion

Conservation of mass:

∂c∂t + ∇·J = S    (I.4)

Fick's Law:

J = −D∇c    (I.5)

Diffusion equation:

∂c∂t = D∇2c + S    (I.6)

## I.4 Charge Transport

Conservation of charge:

∂ρe∂t + ∇·J = 0    (I.7)

Ohm's Law:

J = σE = −σ∇V    (I.8)

# K. Causal Attribution and Underdetermination

## K.1 The Fundamental Problem

Given the product structure:

B = R × D    (K.1)

where B is observed behavior, R is intrinsic response, and D is extrinsic drive, a fundamental ambiguity arises. For any nonzero scalar α:

B = R × D = (αR) × (D/α)    (K.2)

The same behavior B is consistent with infinitely many (R, D) pairs.

## K.2 The Matrix Case

For vector systems J = L·X:

J = L·X = (L·M)·(M−1·X)    (K.3)

for any invertible matrix M. The pair (L·M, M⁻¹·X) produces the same flux J.

## K.3 Resolving Ambiguity

To separate intrinsic from extrinsic contributions, one must:

- Control the drive: Apply known, calibrated forces/fields

- Hold intrinsic properties fixed: Same material, temperature, geometry

- Independent measurement: Measure response coefficients separately

## K.4 Epistemological Implications

The product structure implies:

- Behavior alone cannot support sharp causal attribution

- Auxiliary controls and measurements are always required

- Claims of "pure" intrinsic or extrinsic causation are structurally unjustified

— END OF APPENDIX —