*Defining the force of single spring as,

Fo=koxo

This single spring is subjected to a weight force,

mg=Fo=koxo

So, given the mass attached to the spring and the distance over which it was stretched, the stiffness is,

ko=Foxo=mgxo

And the original displacement is,

xo=Foko=mgko

For two springs of the same stiffness in a parallel configuration supporting the same weight force, the displacement of both springs is the same as the displacement of an equivalent replacement spring,

xpc=xp

The force of the individual springs in parallel,

Fp=koxp

And the force of the equivalent parallel replacement,

Fpc=kpxpc

The weight force is supported by,

mg=Fpc=Fp+Fp

mg=kpxpc=2koxp

Then, the individual stretch of the two springs in parallel is,

xp=mg2ko=xo2

The stiffness of an equivalent parallel replacement is,

kp=2koxpxpc

And since, in parallel we have assumed that xpc=xp, then

kp=2ko

And the stretch of an equivalent parallel replacement (the total stretch of the parallel system),

xpc=mg2ko=2koxp2ko=xp=xo2

For two springs of the same stiffness in a series configuration supporting the same weight force, the displacement of both springs is half that of the displacement of an equivalent replacement spring,

xsc=xs+xs

The force of the individual springs in series,

Fs=koxs

And the force of the equivalent series replacement,

Fsc=ksxsc

The weight force is supported by,

mg=Fsc=Fs

mg=ksxsc=koxs

Then, the individual stretch of the two springs in series is,

xs=mgko=xo

The stiffness of an equivalent series replacement is,

ks=koxsxsc

And since, in parallel we have assumed that xsc=xs+xs, then

ks=koxsxs+xs=koxs2xs=ko2

And the stretch of an equivalent series replacement (the total stretch of the series system),

xsc=xs+xs=2xs=2xo

Summary (For Two Equal Springs Combined in Parallel or Series)

Fo=koxo	Fp=koxp	Fs=koxs	Fpc=kpxpc	      Fsc=ksxsc

xo=Foko		xp=xo2		xs=xo		xpc=xp	xsc=xs+xs

ko=Foxo				kp=2ko			ks=ko2

Fp=koxp=koxo2=Fo2		Fs=koxs=koxo=Fo

Fpc=kpxpc=2koxp=2koxo2=Fo	      Fsc=ksxsc=ko22x0=Fo

In general,

kp=k1+k2+…+kn		ks=k1-1+k2-1+…+kn-1-1

Verifying using a stiffness proportionality:

ko=Foxo=Fpxp=Fsxs

xp=Fpko=FpFoxo=koxpFoxo=1xoxo2xo=xo2

xs=Fsko=FsFoxo=koxsFoxo=1xoxoxo=xo

Now for the combinations,

ko=kp2=Fpc2xpc

xpc=Fpc2ko=Fo2ko=xo2

ko=2ks=2Fscxsc

xsc=2Fscko=2Foko=2xo