On Dynamical Systems

§ Supplementary Section

Thermal Inductance and the Cattaneo–Vernotte Equation

Abstract. Classical thermodynamics models heat conduction as a purely diffusive, first-order process governed by Fourier’s law. Within the bond graph framework, this corresponds to a system with resistance (R) and capacitance (C) elements but no inertance (I) element — a first-order system incapable of oscillation. This section examines whether this absence is a fundamental law of nature or a model limitation. We show that the Cattaneo–Vernotte (CV) hyperbolic heat equation introduces a thermal inertance parameter τⁱ (the heat flux relaxation time), elevates the governing equation to second order, and predicts wave-like heat propagation — a phenomenon experimentally confirmed in several physical systems. The implications for the bond graph analogy framework and for the ODS treatise are discussed.

# 1. The Fourier Model and Its Bond Graph Representation

Fourier's law of heat conduction relates the heat flux vector q to the temperature gradient:

q = −k ∇T   (1)

Combined with conservation of energy, this yields the classical parabolic heat equation:

ρc_p ∂T/∂t = k ∇²T + Q̇   (2)

In bond graph terms, a lumped thermal system contains two passive elements: R (thermal resistance, relating temperature difference to heat flow rate) and C_th (thermal capacitance, relating stored thermal energy to temperature). The governing equation for a single R–C thermal node is:

C_th · dT/dt + T/R_th = Q̇_s   (3)

The characteristic equation s + 1/(R_th C_th) = 0 has a single real negative root. The system is inherently first-order and non-oscillatory. In the cross-domain analogy table, the thermal I element slot is marked “not physical” — but this deserves scrutiny.

# 2. The Cattaneo–Vernotte Equation

Fourier’s law implies that thermal disturbances propagate at infinite speed — a consequence of the parabolic character of Equation (2). This is mathematically convenient but physically untenable: no signal can propagate faster than the speed of light, and at short timescales the thermal equilibration process has a finite relaxation time.

Carlo Cattaneo (1948) and Pieter Vernotte (1958) independently proposed a modification to Fourier’s law that introduces a heat flux relaxation time τⁱ:

τ_q ∂q̇/∂t + q = −k∇T   (4)

This is the Cattaneo–Vernotte (CV) constitutive law. The term τⁱ ∂q/∂t asserts that heat flux cannot change instantaneously — it has inertia. When this replaces Fourier’s law in the energy equation, the result is the hyperbolic heat equation:

τ_q ∂²T/∂t² + ∂T/∂t = α ∇²T   (5)

where α = k/(ρc_p) is the thermal diffusivity. Equation (5) is a telegraph equation — the canonical form of a damped wave equation. It admits solutions of the form T(x,t) = e^{−αt} cos(kx − ωt), describing temperature waves that propagate at finite speed c = √(α/τⁱ) and decay exponentially in time.

## 2.1 The Thermal Inertance Parameter

In lumped form, the CV equation for a single node becomes:

C_th τ_q · d²T/dt² + C_th · dT/dt + T/R_th = Q̇_s   (6)

Comparing with the standard second-order bond graph form I·d²q/dt² + R·dq/dt + (1/C)·q = e, the identification is:

| Bond Graph | Symbol | Thermal (Fourier) | Thermal (CV) |
| --- | --- | --- | --- |
| I (inertance) | τ_q · C_th | — (absent) | Present |
| C (capacitance) | C_th | C_th = ρc_p V | C_th = ρc_p V |
| R (resistance) | R_th | R_th = L/(kA) | R_th = L/(kA) |
| System order |  | 1st order | 2nd order |

Table 1. Bond graph element mapping for classical Fourier and Cattaneo–Vernotte thermal models.

The thermal inertance in the CV model is not an independent parameter — it is I_th = τ_q · C_th. The single new physical quantity is the heat flux relaxation time τⁱ. The natural frequency and damping ratio of the CV thermal system are:

ω_n = 1/√(I_th · C_th) = 1/τ_q   (7)

ζ = (R_th/2) · √(C_th / I_th) = R_th C_th / (2τ_q)   (8)

# 3. Experimental Evidence for Thermal Inertance

Thermal wave propagation is not theoretical — it has been directly observed in several systems where τⁱ is large enough to be measurable:

- Superfluid Helium-4 below T_λ = 2.17 K

  - Thermal waves (“second sound”) propagate at ~20 m/s. First observed by Peshkov (1944). The clearest macroscopic demonstration of a finite heat flux relaxation time.

- Solid Helium-3 and Helium-4

  - Second sound observed in solid phases, demonstrating that thermal inertance persists into crystalline states.

- Sodium fluoride (NaF) crystals near 10 K

  - Jackson and Walker (1971) observed thermal pulses propagating as waves with τ_q on the order of 10⁻¹ s — measurable with standard electronics.

- Bismuth at cryogenic temperatures

  - Thermal wave propagation observed by Narayanamurti and Dynes (1972).

- Graphene and carbon nanotubes at room temperature

  - At nanoscale dimensions and picosecond timescales, phonon mean free paths exceed system dimensions and τ_q becomes significant. Active area of current research in nanoscale heat transfer.

In all cases, the observability of thermal waves requires either very low temperatures (where phonon–phonon scattering is suppressed and τ_q increases), or very small length scales and short timescales (where the phonon mean free path Λ becomes comparable to the system dimension L). The relevant dimensionless parameter is the Knudsen number Kn = Λ/L. When Kn ≪ 1, Fourier’s law applies. When Kn ≈ 1 or Kn > 1, the CV model (or full phonon Boltzmann transport) is required.

# 4. Implications for the Bond Graph Analogy Framework

The thermal I element is not absent from nature — it is absent from the engineering scale. This distinction matters for the ODS treatise in three ways.

## 4.1 The Analogy Is Scale-Dependent

The standard claim that “thermal systems have no I element” should be qualified: thermal inertance is negligible when τ_q ≪ R_th C_th, which holds for all conventional engineering applications (HVAC, heat exchangers, electronic cooling). The Fourier first-order model is then an excellent approximation. At nanoscale or cryogenic conditions, τ_q becomes comparable to or larger than the system’s thermal time constant, and the full second-order CV model applies.

This is analogous to how the lumped capacitance model (Bi < 0.1) breaks down when spatial temperature gradients become significant — the model is not wrong, it has a domain of validity.

## 4.2 The Bond Graph Is More General Than the Fourier Model

The bond graph framework, by admitting I elements in the thermal domain, is actually more general than Fourier’s law. A bond graph with a thermal I element is a valid physical model — it describes the CV system. The PhysicaFlow simulator should therefore allow users to add I elements to thermal systems and should display the resulting second-order behavior, with the annotation that the I value represents τ_q · C_th and is physically relevant only at nanoscale or cryogenic conditions.

## 4.3 Connection to the ODS Underdetermination Theorem

The ODS treatise establishes that the product structure B = R × D is underdetermined — the same bond graph topology admits multiple physical interpretations. The thermal I example is a concrete instance: the abstract inertance element I can be interpreted as a mass, an inductor, a fluid inertance, or τ_q C_th depending on domain. The fact that thermal inertance requires extreme conditions to be observable does not undermine the analogy — it reveals that physical realizability is a constraint on the domain D, not on the abstract bond graph structure B.

# 5. Summary

| Question | Fourier Model | Cattaneo–Vernotte Model |
| --- | --- | --- |
| Governing PDE type | Parabolic | Hyperbolic |
| Propagation speed | Infinite | c = √(α/τ_q) (finite) |
| Bond graph order | 1st order (R–C) | 2nd order (I–R–C) |
| Oscillation possible? | No | Yes (second sound) |
| Domain of validity | Engineering scales (Kn ≪ 1) | Nanoscale, cryogenic (Kn ≈ 1) |
| Thermal I element | Absent / negligible | τ_q · C_th |

Table 2. Comparison of Fourier and Cattaneo–Vernotte thermal models.

The thermal I element exists physically — it is the heat flux relaxation time τ_q multiplied by the thermal capacitance. Its absence from standard engineering models reflects a scale assumption (Kn ≪ 1), not a law of nature. The bond graph framework, by treating I as a valid thermal element, is implicitly more general than Fourier’s law and correctly describes wave-like heat conduction when τ_q is non-negligible. This is precisely the kind of cross-domain structural insight the ODS treatise aims to formalize.

# References

[1]  Cattaneo, C. (1948). Sulla conduzione del calore. Atti del Seminario Matematico e Fisico dell’Università di Modena, 3, 83–101.

[2]  Vernotte, P. (1958). Les paradoxes de la théorie continue de l’équation de la chaleur. Comptes Rendus Hebdomadaires des Séances de l’Académie des Sciences, 246, 3154–3155.

[3]  Peshkov, V. (1944). Second sound in helium II. Journal of Physics USSR, 8, 381.

[4]  Jackson, H. E., & Walker, C. T. (1971). Thermal conductivity, second sound, and phonon–phonon interactions in NaF. Physical Review B, 3(4), 1428.

[5]  Narayanamurti, V., & Dynes, R. C. (1972). Observation of second sound in bismuth. Physical Review Letters, 28(22), 1461.

[6]  Joseph, D. D., & Preziosi, L. (1989). Heat waves. Reviews of Modern Physics, 61(1), 41–73.

[7]  Karnopp, D. C., Margolis, D. L., & Rosenberg, R. C. (2012). System Dynamics: Modeling, Simulation, and Control of Mechatronic Systems (5th ed.). Wiley.

[8]  Tzou, D. Y. (1997). Macro- to Micro-Scale Heat Transfer: The Lagging Behavior. Taylor & Francis.